# Opus 4.6

Run 1 — REVISED with Full Dataset Context

Now with the complete data dictionary applied, here is the fully supplemented analysis:

📊 Prompt #1 — REVISED Data Discovery

Dataset Overview

| Property | Details |
| --- | --- |
| File Name | FTES-Full_Test_1hour_avg.csv |
| System | Demo-FTES Test 1 (Subsurface Thermal Energy Storage) |
| Date Range | December 10, 2024 → March 24, 2025 |
| Granularity | 1-hour averages |
| Wells Monitored | TL, TN, TC, TU, TS |
| Measurement Points | Collar, Interval, Bottom (per well) |
| Units | Temp=°C, Pressure=psi, Flow=L/min, EC=µS/cm, Depth=Feet |

Two Main Operational Phases

| Phase | Period | Description |
| --- | --- | --- |
| Phase 1 - Hot Injection | Dec 13, 2024 20:00 → Feb 23, 2025 11:00 | Hot water injected via TC, produced from TN & TL |
| Phase 2 - Ambient Injection | Mar 12, 2025 13:00 → Mar 24, 2025 22:00 | Ambient water injected via TN & TS, produced from TC |
| In-Between | Various | Testing & maintenance periods |

🗂️ Column Identification — Now Named

System-Level Columns

| Column Position | Column Name | Description |
| --- | --- | --- |
| 1 | Time | Timestamp |
| 2 | Triplex On/Off | Triplex pump status |
| 3 | Net Flow | Injection flow rate (L/min) |

Per-Well Measurements (×5 wells: TL, TN, TC, TU, TS)

Each well contains:

| Measurement Type | Description | Units |
| --- | --- | --- |
| Interval Flow | Production flow rate at interval | L/min |
| Bottom Flow | Production flow rate at bottom | L/min |
| Collar Flow | Production flow rate at collar | L/min |
| Interval EC | Electrical conductivity at interval | µS/cm |
| Bottom EC | Electrical conductivity at bottom | µS/cm |
| Interval Pressure | Water pressure at interval | psi |
| Bottom Pressure | Water pressure at bottom | psi |
| Packer Pressure | Packer pressure | psi |
| TEC-INT-U | Temperature interval upper | °C |
| TEC-INT-L | Temperature interval lower | °C |
| TEC-BOT-U | Temperature bottom upper | °C |
| TEC-BOT-L | Temperature bottom lower | °C |

Depth & Boolean Columns

| Column Name | Description |
| --- | --- |
| TX Packer Center Depth | Depth along borehole of packer (Feet) |
| TX injecting triplex interval | True/False — is TX injecting via triplex? |
| TX producing interval | True/False — is TX interval open for production? |
| TX producing bottom | True/False — is TX bottom open for production? |

⚠️ Prompt #2 — REVISED Bias & Limitations

Updated Bias Assessment

1. Phase Transition Bias

- Data spans two fundamentally different operational phases

- Hot injection vs. ambient injection will create bimodal distributions in temperature, pressure, and flow features

- Any ML model must account for this phase label or risk learning spurious correlations

2. Injection Well Role Switching

- TC is the injector in Phase 1, then becomes a producer in Phase 2

- TN is a producer in Phase 1, then becomes an injector in Phase 2

- Boolean columns (TX injecting triplex interval) capture this — must be used as features

3. Maintenance Period Gaps

- Periods before and between phases are testing/maintenance

- These records may contain non-representative readings and should be flagged or excluded

4. Sensor Location Bias

- Collar, interval, and bottom measurements reflect physically different zones

- Collar ≠ Interval ≠ Bottom — mixing these without zone labels could mislead models

5. Packer Depth Variability

- TX Packer Center Depth may shift slightly over time

- Depth changes could affect pressure readings without indicating a true process change

🧹 Prompt #3 — REVISED Data Quality Check

Specific Issues Now Identified

| Issue | Specific Columns Affected | Recommendation |
| --- | --- | --- |
| Constant last 4 values | 170.2, 133.6, 142.0, 177.8 | These are Packer Center Depths (TL, TN, TC, TU, TS) — confirm if static is expected |
| Zero final column | Last column = 0.0 consistently | Likely a boolean production flag = False during this period |
| Phase transition gaps | Net Flow, TC/TN Interval Flow | Flag rows between Feb 23 and Mar 12 as maintenance |
| Injector/producer role swap | All TC & TN flow columns | Split analysis by phase or use boolean flags as covariates |
| Small EC values (~0.001–0.007) | Interval EC, Bottom EC | Verify units — may indicate low-mineralization water or sensor noise |
| Large pressure range (434–5499 psi) | All pressure columns | Confirm range is expected across injection vs. production wells |

🎯 Prompt #4 — REVISED Problem Statements

Option A — Thermal Recovery Prediction ⭐ Recommended

"Can we predict the thermal energy recovery rate (based on produced water temperature at TN/TL intervals) during Phase 1 hot injection using flow rate, pressure, and EC as input features?"

- Target: TN-TEC-INT-U or TL-TEC-INT-U (produced temperature)

- Value: Optimize hot water injection strategy for FTES systems

Option B — Operational Phase Classification

"Can we classify whether the FTES system is in hot injection, ambient injection, or maintenance phase using only sensor readings (excluding boolean flags)?"

- Target: Operational phase label (3-class)

- Value: Automated state detection from sensor data alone

Option C — Anomaly Detection During Injection

"Can we detect anomalous pressure or flow events during triplex pump operation that may indicate packer failure or formation changes?"

- Target: Binary anomaly flag

- Value: Predictive maintenance for borehole integrity

🚨 Prompt #7 — REVISED LLM Error Watch List

Errors to Actively Check When Running on Other Models

| Error Type | Specific Risk for FTES Data |
| --- | --- |
| Phase confusion | LLM may not distinguish TC as injector vs. producer across phases |
| Unit hallucination | LLM may report pressure in kPa instead of psi |
| Depth treated as dynamic | LLM may flag constant packer depth values as anomalies when they are expected |
| Boolean misinterpretation | LLM may treat True/False columns as numerical 1/0 without noting operational meaning |
| EC units wrong | LLM may misreport EC units or confuse EC with another measurement |

📋 Run 1 — FINAL Summary Log

| Field | Details |
| --- | --- |
| LLM Used | Claude (this session) |
| Dataset | FTES-Full_Test_1hour_avg.csv |
| Context Applied | FTES_data_description.docx ✅ |
| Columns Now Named | ✅ All 65+ columns identified |
| Phases Identified | ✅ Phase 1 (Hot) & Phase 2 (Ambient) |
| Units Confirmed | °C, psi, L/min, Feet |
| Errors Flagged | 6 data quality issues |
| Best Problem Statement | Thermal Recovery Prediction (Option A) |
| Next Step | Run identical prompts on ChatGPT & Gemini for comparison |

# Gemini 2.5 (pro)

Run 1 — REVISED with Full Dataset Context

This analysis has been updated with the column headers and sample data rows, providing a much more accurate and detailed assessment.

📊 Prompt #1 — REVISED Data Discovery

Dataset Overview

| Property | Details |
| --- | --- |
| System | Subsurface Thermal Energy Storage (FTES) |
| Wells Monitored | TL, TN, TC, TU, TS |
| Total Columns | 69 |
| Data Types | Timestamp, 68 numerical features (float) |
| Units | Temp (°C), Pressure (psi), Flow (L/min), EC (µS/cm), Depth (Feet) |
| Key Observation | The data shows at least two distinct operational phases, visible in the dramatic shifts in pressure and flow values between January/February and March samples. |

Column Groups

| Group | Key Columns | Description |
| --- | --- | --- |
| System-Level | Time, Injection EC, Net Flow, Injection Pressure | Overall metrics for the injection process. |
| Well Monitoring | [Well] Interval Flow, [Well] Bottom Pressure, [Well]-TEC-INT-U | Per-well measurements for flow, pressure, electrical conductivity (EC), and temperature at different points (interval, bottom). |
| Packer Depths | TL Packer Center Depth ... TS Packer Center Depth | Depths for the packers in each of the five wells. These values appear to change between operational phases, not static. |

⚠️ Prompt #2 — REVISED Bias & Limitations

Updated Bias & Limitations Assessment

1. Bi-Modal Operational Phases (Major Factor)

- The data contains at least two phases: a high-pressure injection phase (visible in Jan/Feb data where TC Interval Pressure is ~5000+ psi) and a low-pressure phase (visible in March data where TC Interval Pressure is ~3 psi).

- A model trained on the entire dataset without accounting for these phases will perform poorly. The underlying physical processes are different.

2. Role-Switching Wells

- The pressure and flow data strongly suggest that wells switch roles.

- Well TC is an injector in the high-pressure phase and likely a producer/monitor in the low-pressure phase.

- Well TN is a producer/monitor in the high-pressure phase and appears to become an injector in the low-pressure phase (its pressure increases significantly). This must be accounted for.

3. Inconsistent Packer Depths

- The [Well] Packer Center Depth columns are not static. For example, TL Packer Center Depth is 170.2 in one sample but 149.0 in another.

- This indicates physical changes to the system setup between phases. Any model relating pressure to other variables must include packer depth as a feature.

4. Data Gaps Between Phases

- The provided samples jump from February to March, implying gaps. These gaps likely correspond to maintenance or system reconfiguration periods and contain non-standard data that should be excluded from operational modeling.

🧹 Prompt #3 — REVISED Data Quality Check

Specific Data Quality Issues Identified from Samples

| Issue | Specific Column & Value | Recommendation |
| --- | --- | --- |
| Suspect Negative Values | TC Bottom EC shows a value of -500.009. | This is physically impossible for electrical conductivity. This is likely a sensor error code or a placeholder for a null value. It must be cleaned (e.g., imputed or the row dropped). |
| Negative Pressure | TN Interval Pressure and others show negative values (e.g., -11.21 psi). | This could be a calibration issue or another error code. Investigate if sub-atmospheric (vacuum) pressures are possible in the system; if not, treat as a data quality error. |
| Anomalous 0.0 Depth | In one sample, TS Packer Center Depth is 0.0 while others are >100. | This indicates the TS well was likely not configured or used in that phase. This should be treated as a categorical state (inactive) rather than a numerical zero. |
| Out-of-Order Data | The raw file may not be sorted by time, as seen in the initial exploration. | Crucial first step: Sort the entire dataset by the Time column before any time-series analysis. |

🎯 Prompt #4 — REVISED Problem Statements

Option A — Thermal Efficiency Prediction (Recommended) ⭐

"During the hot injection phase (where TC is the injector), can we predict the temperature of the produced water at a monitoring well (e.g., TN-TEC-INT-U) based on injection parameters (Injection Pressure, Net Flow) and packer depths?"

- Target Variable: TN-TEC-INT-U (or another production temperature).

- Value: Optimizing energy storage and retrieval by understanding thermal breakthrough and efficiency.

- Data Subset: Use only data from the "Hot Injection" phase.

Option B — Anomaly Detection Model

"Can we build a model to automatically detect sensor failures or non-physical readings, such as the negative EC value (-500.009) or anomalous pressure readings?"

- Task: Anomaly detection (unsupervised or supervised).

- Value: Improve data integrity and provide real-time alerts for system maintenance.

Option C — Operational Phase Classifier

"Can we train a classification model to automatically label each time-step as 'Hot Injection', 'Ambient Injection', or 'Maintenance' using only the sensor pressure, flow, and temperature data?"

- Target Variable: A categorical "Phase" label.

- Value: Automating the segmentation of the dataset for more accurate phase-specific analysis.

🚨 Prompt #7 — REVISED LLM Error Watch List

When running these prompts on other LLMs, watch for these specific, now-contextualized errors:

| Potential Error Type | Specific Example to Watch For |
| --- | --- |
| Phase Ignorance | The LLM provides a single summary (e.g., mean pressure) for the entire dataset, ignoring the massive difference between the injection and production phases. |
| Misinterpreting Packer Depth | The LLM might flag the changing Packer Center Depth values as data drift or an anomaly, when it's actually a planned system reconfiguration. |
| Ignoring Error Codes | The LLM might include the -500.009 EC value in its statistical calculations (e.g., min, mean), skewing the results, instead of identifying it as an error. |
| Role Confusion | The LLM may fail to state that Well TC's role changes from an injector to a producer, a critical piece of context for any analysis. |